| Project Title: | B-BOT: WiFi-enabled self-balancing wheel-legged robot |  | Submission Date: | 25 April 2026 |  |
| --- | --- | --- | --- | --- | --- |
| Student Name: | Botao Su |  |  |  |  |

| Project Risk | Severity |  |  | Potential |  |  | Score (Severity x Potential) | Mitigation Measures |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | L | M | H | L | M | H | L=1, M=2, H=3 |  |
| High-torque wheel or leg motors may pinch fingers, catch clothing, or move unexpectedly during practical testing. |  |  | X |  | X |  | 6 | Keep hands, clothing and tools clear of wheels and joints while powered. Make mechanical adjustments only when motors are disabled or power is isolated. Start with low-energy bench tests before free-standing tests. |
| The self-balancing robot may fall during free-standing balance, disturbance, leg-length, jump or cross-step tests. |  |  | X |  | X |  | 6 | Use a clear test area, keep bystanders away, begin with the robot supported or restrained, avoid standing in the fall path, and keep power isolation within reach. Record failed trials rather than repeating unsafe attempts. |
| Battery, wiring or PCB fault may cause short circuit, overheating, smoke, or damaged electronics during integration. |  |  | X |  | X |  | 6 | Inspect polarity and insulation before power-up, avoid exposed conductors, use conservative bench power where possible, stop immediately if heat/smell is detected, and do not leave powered hardware unattended. |
| Soldering PCB, wiring harnesses and motor leads may cause burns or fume exposure. |  | X |  |  | X |  | 4 | Use an iron stand, work in a ventilated area, avoid touching recently soldered joints, keep cables supported while cooling, and follow lab soldering practice. |
| WiFi, BLE or vision-generated commands may trigger unexpected motion during software or presentation testing. |  |  | X |  | X |  | 6 | Use dry-run mode for initial vision tests, secure the robot when testing command generation, keep stunt actions disabled unless explicitly armed, rely on watchdogs/full-stop behaviour, and test stop commands before motion. |
| Sharp 3D-printed parts, PCB edges or cut wires may cause minor cuts during assembly and modification. |  | X |  | X |  |  | 2 | Deburr or file sharp edges, trim wire ends, handle PCBs by edges, and use appropriate hand protection when modifying parts. |
| USB, power or camera cables may create trip hazards or pull the robot from the bench during debugging. |  | X |  | X |  |  | 2 | Route cables away from walkways, keep slack controlled, avoid testing near bench edges, and disconnect unnecessary cables before moving tests. |
| Eye/face injury or impact may occur if a moving robot, wheel, printed part, or loose connector fails during dynamic tests. |  |  | X | X |  |  | 3 | Check mechanical fasteners and wiring before tests, avoid high-energy stunt tests unless secured, maintain safe distance, and stop testing if vibration or loose parts are observed. |
| Fatigue and time pressure near the report deadline may lead to rushed or unsafe practical testing. |  | X |  |  | X |  | 4 | Use written test procedures, define stop criteria before testing, avoid late-night risky hardware tests, and prioritise safe repeated measurements over risky demonstrations. |
| Loss of experiment data, code versions, or planning evidence may reduce project traceability and encourage repeated tests. |  | X |  |  | X |  | 4 | Commit code and report changes to Git, keep raw CSV logs and generated figures, record provisional data clearly, and maintain Progress.md and appendix evidence after major work. |